OpenShift Agent-Based Cluster

Installation Guide

Step-by-Step Build Process

OCP 4.19 | Agent-Based Installer | lab.kuberox.net

# Overview

This guide documents the complete process to build an OpenShift (OCP) 4.19 cluster using the Agent-Based Installer method. The agent-based approach boots each node from a live ISO image that contains the installer agent, eliminating the need for a PXE/DHCP infrastructure.

The bastion host (ocp-bastion-shivang.lab.kuberox.net) acts as the DNS, load balancer, and HTTP file server. All cluster nodes pull configuration from the ISO generated on the bastion.

## Architecture Summary

The following services are configured on the bastion host:

- HAProxy – Layer-4 load balancer for API (6443), Machine Config (22623), and Ingress (80/443) traffic

- BIND (named) – Authoritative DNS for forward and reverse zones in lab.kuberox.net

- Apache HTTPD – HTTP server to host the agent ISO for node download (optional)

- openshift-install – Generates install-config.yaml + agent-config.yaml and builds the bootable ISO

# Phase 1 – Install & Configure HAProxy

## 1.1  Install HAProxy

yum install haproxy -y cp /etc/haproxy/haproxy.cfg /etc/haproxy/haproxy.cfg.bkp

📝 Note: Always backup the original config before editing.

## 1.2  Configure HAProxy

Edit /etc/haproxy/haproxy.cfg and define frontends and backends for each OCP endpoint:

| # | Command | Purpose |
| --- | --- | --- |
| 1 | vi /etc/haproxy/haproxy.cfg | Edit load balancer config (API :6443, MCS :22623, HTTP :80, HTTPS :443) |
| 2 | haproxy -c -f /etc/haproxy/haproxy.cfg | Validate config syntax before starting the service |
| 3 | systemctl enable --now haproxy | Enable and start HAProxy |
| 4 | systemctl status haproxy.service | Verify HAProxy is active and running |

## 1.3  Open Firewall Ports

firewall-cmd --add-port={80,443,6443,22623}/tcp --permanent firewall-cmd --add-port=8080/tcp --permanent firewall-cmd --reload firewall-cmd --list-all

📝 Note: Port 8080 may be required if HTTPD serves the ISO on a non-standard port.

## 1.4  Fix SELinux Port Labeling

If SELinux is Enforcing, HAProxy cannot bind to non-standard ports unless they are labeled correctly:

semanage port -a -t http_port_t -p tcp 443 semanage port -a -t http_port_t -p tcp 80 semanage port -a -t http_port_t -p tcp 8080 semanage port -a -t http_port_t -p tcp 6443 semanage port -a -t http_port_t -p tcp 22623

⚠️  Warning: Do NOT use setenforce 0 in production. Properly label the ports instead.

systemctl restart haproxy systemctl status haproxy.service

# Phase 2 – Install & Configure BIND DNS

## 2.1  Install BIND

yum install bind -y

## 2.2  Configure named.conf

Edit the main BIND configuration file to allow queries from your lab network and define zone references:

vi /etc/named.conf

Key settings to verify/add inside named.conf:

- listen-on port 53 – include the bastion IP

- allow-query – set to your lab subnet (e.g. 190.170.20.0/24)

- forwarders – point to an upstream DNS if needed

## 2.3  Define Forward and Reverse Zones

vi /etc/named.rfc1912.zones

Add zone stanzas for the forward and reverse lookup zones. Example:

zone "lab.kuberox.net" IN { type master; file "local.for"; allow-update { none; }; }; zone "20.170.190.in-addr.arpa" IN { type master; file "local.rev"; allow-update { none; }; };

## 2.4  Create Zone Files

cd /var/named/ cp -p named.localhost local.for cp named.loopback local.rev

Edit the forward zone (local.for). Ensure A records exist for all nodes:

vi local.for # Add A records for bastion, masters, workers, api, api-int, *.apps

Minimum required DNS records for OCP:

- api.<cluster>.<base_domain>  →  bastion / load balancer IP

- api-int.<cluster>.<base_domain>  →  bastion / load balancer IP

- *.apps.<cluster>.<base_domain>  →  ingress/worker IPs

- master-shivang-01.ocp.lab.kuberox.net  →  master node IP

- ocp-bastion-shivang.lab.kuberox.net  →  bastion IP

Edit the reverse zone (local.rev). Add PTR records:

vi local.rev # Add PTR records for each IP

## 2.5  Set Correct Permissions & Start Named

chown root:named /var/named/local.rev chmod 640 /var/named/local.rev systemctl enable --now named systemctl restart named systemctl status named

## 2.6  Open DNS Firewall Port

firewall-cmd --add-service=dns --permanent firewall-cmd --reload

## 2.7  Validate DNS Resolution

Run the following lookups to verify forward and reverse DNS are working:

nslookup ocp-bastion-shivang.lab.kuberox.net nslookup master-shivang-01.ocp.lab.kuberox.net nslookup 190.170.20.181 nslookup 190.170.20.182 nslookup 190.170.20.183 nslookup 190.170.20.244

⚠️  Warning: ALL forward and reverse lookups must succeed before proceeding. The installer will fail if DNS is broken.

# Phase 3 – Configure Bastion as DNS Resolver

Once named is running and validated, configure the bastion host itself to use its own DNS:

nmtui # Set DNS server to 127.0.0.1 (or bastion IP) in the network connection

After applying, verify the bastion resolves via its own DNS:

nslookup ocp-bastion-shivang.lab.kuberox.net

📝 Note: The bastion must resolve cluster names using its own BIND instance — not an external DNS — because the cluster FQDNs only exist locally.

# Phase 4 – Install OpenShift CLI Binaries

## 4.1  Download Binaries

mkdir ~/binaries && cd ~/binaries wget https://mirror.openshift.com/pub/openshift-v4/clients/ocp/4.19.20/openshift-client-linux-4.19.20.tar.gz wget https://mirror.openshift.com/pub/openshift-v4/clients/ocp/4.19.20/openshift-install-linux-4.19.20.tar.gz

## 4.2  Extract and Install Binaries

tar -xvzf openshift-client-linux-4.19.20.tar.gz tar -xvzf openshift-install-linux-4.19.20.tar.gz cp kubectl oc openshift-install /usr/local/bin

## 4.3  Verify Versions

oc version kubectl version openshift-install version

# Phase 5 – Prepare Agent Installer Directory

## 5.1  Create Working Directory

mkdir ~/agent-4.19.20 && cd ~/agent-4.19.20

## 5.2  Generate SSH Key

This key will be injected into all cluster nodes for post-install access:

ssh-keygen # Accept defaults, creates ~/.ssh/id_rsa and id_rsa.pub cat ~/.ssh/id_rsa.pub # Copy this value into install-config.yaml

## 5.3  Obtain Pull Secret

Download your pull secret from https://console.redhat.com/openshift/install/pull-secret and save it:

vi pull-secret # Paste the JSON content of your Red Hat pull secret

# Phase 6 – Create install-config.yaml

This file defines the cluster topology. Create it inside ~/agent-4.19.20/:

vi install-config.yaml

Minimum required fields:

apiVersion: v1 baseDomain: lab.kuberox.net metadata: name: ocp networking: networkType: OVNKubernetes clusterNetwork: - cidr: 10.128.0.0/14 hostPrefix: 23 serviceNetwork: - 172.30.0.0/16 compute: - name: worker replicas: 0 # 0 for compact/SNO; set actual count for workers controlPlane: name: master replicas: 3 platform: baremetal: {} # or none: {} for agnostic pullSecret: '<paste pull secret here>' sshKey: '<paste id_rsa.pub here>'

📝 Note: Always backup the config: cp install-config.yaml install-config.yaml.bkp — the installer DELETES this file during generation.

cp install-config.yaml install-config.yaml.bkp

# Phase 7 – Create agent-config.yaml

This file maps node hardware (MAC addresses / IPs) to their cluster roles:

vi agent-config.yaml

Example structure:

apiVersion: v1alpha1 kind: AgentConfig metadata: name: ocp rendezvousIP: 190.170.20.181 # IP of the bootstrap/first master hosts: - hostname: master-shivang-01 role: master interfaces: - name: ens3 macAddress: 'AA:BB:CC:DD:EE:01' networkConfig: interfaces: - name: ens3 type: ethernet state: up ipv4: enabled: true address: - ip: 190.170.20.181 prefix-length: 24 dhcp: false dns-resolver: config: server: - 190.170.20.244 routes: config: - destination: 0.0.0.0/0 next-hop-address: 190.170.20.1 next-hop-interface: ens3

📝 Note: Repeat the hosts block for each master and worker node. The rendezvousIP must be reachable from all other nodes.

cp agent-config.yaml agent-config.yaml.bkp

# Phase 8 – Install nmstate Dependency

The agent-config.yaml uses NMState networking syntax. The installer requires the nmstate library to process it:

yum install nmstate -y

# Phase 9 – Generate the Agent ISO

cd ~/agent-4.19.20/ openshift-install --dir . agent create image

On success, the installer produces:

- agent.x86_64.iso – bootable ISO containing the agent, config, and NMState manifests

- .openshift_install.log – detailed log for troubleshooting

📝 Note: The process downloads RHCOS and OCP release images from the internet. Ensure the bastion has internet access or configure a mirror registry.

# Phase 10 – Serve the ISO via HTTPD

Host the ISO so nodes can optionally download it via HTTP (useful for virtual environments or iDRAC virtual media):

yum install httpd -y systemctl enable --now httpd mkdir -p /var/www/html/agent cp ~/agent-4.19.20/agent.x86_64.iso /var/www/html/agent/

Verify it is accessible from another host:

curl -I http://190.170.20.244:8080/agent/agent.x86_64.iso

# Phase 11 – Boot Nodes from ISO & Monitor Installation

## 11.1  Boot Each Node

Mount or attach the agent.x86_64.iso to each cluster node (virtual CD-ROM, iDRAC virtual media, USB, etc.) and boot from it. The agent will:

- Apply the NMState network configuration from agent-config.yaml

- Connect to the rendezvous IP and begin cluster bootstrapping

- Download and install RHCOS to local disk

## 11.2  Monitor Installation Progress

From the bastion, monitor the installation with:

cd ~/agent-4.19.20/ openshift-install --dir . agent wait-for bootstrap-complete --log-level=info openshift-install --dir . agent wait-for install-complete --log-level=info

📝 Note: Bootstrap takes ~15–20 minutes; full install takes 30–60 minutes depending on hardware.

## 11.3  Access the Cluster

export KUBECONFIG=~/agent-4.19.20/auth/kubeconfig oc get nodes oc get clusteroperators

# Troubleshooting Reference

| # | Command | Purpose |
| --- | --- | --- |
| 1 | systemctl status haproxy | Check HAProxy is running |
| 2 | systemctl status named | Check DNS service health |
| 3 | journalctl -u named -xe | View BIND error logs |
| 4 | nslookup api.ocp.lab.kuberox.net | Test API DNS record |
| 5 | semanage port -l ¦ grep http | Verify SELinux port labels |
| 6 | firewall-cmd --list-all | Review active firewall rules |
| 7 | openshift-install --dir . agent wait-for bootstrap-complete | Monitor bootstrap phase |
| 8 | oc get co | Check cluster operator status post-install |

# Quick Reference – Key Files & Paths

| File / Path | Purpose |
| --- | --- |
| /etc/haproxy/haproxy.cfg | HAProxy load balancer configuration |
| /etc/named.conf | BIND main configuration |
| /etc/named.rfc1912.zones | Zone definitions |
| /var/named/local.for | Forward DNS zone file |
| /var/named/local.rev | Reverse DNS zone file |
| ~/agent-4.19.20/install-config.yaml | Cluster topology definition |
| ~/agent-4.19.20/agent-config.yaml | Node/NIC/IP mapping config |
| ~/agent-4.19.20/agent.x86_64.iso | Bootable agent ISO |
| /var/www/html/agent/ | HTTPD directory for ISO hosting |
| ~/agent-4.19.20/auth/kubeconfig | kubeconfig for cluster access |